## Appendix

## Sensitivity Analysis of Key Hyperparameters

To further verify the rationality of the key hyperparameter settings in the DLE model, one-factor-at-a-time sensitivity analyses were conducted on the Caltech and JPL datasets. Specifically, the effects of the adjustment-factor control parameter, kurtosis weighting coefficient, skewness weighting coefficientand warm-up roundson model performance were examined. In each experiment, only one parameter was varied while the remaining parameters were kept at their default values. The results are reported as the mean value and maximum deviation across the six training windows.

As shown in Table S-I, on the Caltech dataset, when,, or, the parking-duration MAE decreases from 0.89 h under the default setting to approximately 0.84 h. However, the corresponding MAE and SMAPE for charging energy increase to varying degrees. For example, when, the charging-energy MAE increases from 1.35 kWh to 1.37 kWh. Table S-II shows that the JPL dataset exhibits a similar trend. Some parameter settings slightly reduce the parking-duration error; for instance, when, the parking-duration MAE decreases to 0.90 h. Nevertheless, the charging-energy MAE increases to 2.95 kWh, which is inferior to the 2.88 kWh obtained under the default parameter setting.

Considering the results across the two prediction targets of the two datasets, the default parameter combination does not necessarily yield the minimum value for every individual metric, but it provides a more balanced performance between parking-duration and charging-energy prediction. In particular, for charging-energy prediction, this combination maintains relatively low errors on both the Caltech and JPL datasets, while the parking-duration errors remain close to the locally optimal results. Therefore,,,, andare selected as the default hyperparameter settings to balance prediction accuracy and stability across the two targets.

TABLE  S-I

Sensitivity Analysis Results of Key Hyperparameters for the DLE Model on the Caltech Dataset

| Parameter | Parameter Setting | Parking Duration SMAPE (%) | Parking Duration MAE (h) | Charging Capacity SMAPE (%) | Charging Capacity MAE (kWh) |
| --- | --- | --- | --- | --- | --- |
| Baseline | Default | 10.41±0.61 | 0.89±0.06 | 6.71±2.35 | 1.35±0.44 |
|  | 0.1 | 10.08±0.82 | 0.84±0.09 | 6.75±2.60 | 1.36±0.49 |
|  | 0.3 | 10.14±0.70 | 0.85±0.08 | 6.82±2.66 | 1.37±0.50 |
|  | 0.5 | 10.20±0.77 | 0.85±0.09 | 6.84±2.50 | 1.38±0.47 |
|  | 0.7 | 10.41±0.61 | 0.89±0.06 | 6.71±2.35 | 1.35±0.44 |
|  | 0.9 | 10.20±0.77 | 0.85±0.09 | 6.83±2.11 | 1.38±0.39 |
|  | 1 | 10.17±0.74 | 0.85±0.09 | 6.80±2.38 | 1.37±0.44 |
|  | 3 | 10.20±0.76 | 0.85±0.09 | 6.83±2.44 | 1.38±0.45 |
|  | 5 | 10.41±0.61 | 0.89±0.06 | 6.71±2.35 | 1.35±0.44 |
|  | 7 | 10.21±0.75 | 0.85±0.09 | 6.76±2.16 | 1.36±0.40 |
|  | 9 | 10.22±0.83 | 0.85±0.10 | 6.82±2.22 | 1.37±0.41 |
|  | 1 | 10.19±0.70 | 0.85±0.08 | 6.85±2.43 | 1.38±0.45 |
|  | 3 | 10.19±0.70 | 0.85±0.08 | 6.84±2.45 | 1.38±0.46 |
|  | 5 | 10.41±0.61 | 0.89±0.06 | 6.71±2.35 | 1.35±0.44 |
|  | 7 | 10.20±0.77 | 0.85±0.09 | 6.75±2.14 | 1.36±0.39 |
|  | 9 | 10.20±0.76 | 0.85±0.09 | 6.81±2.29 | 1.37±0.42 |
|  | 5 | 10.23±0.75 | 0.86±0.09 | 6.75±1.98 | 1.36±0.36 |
|  | 10 | 10.25±0.72 | 0.86±0.09 | 6.80±2.13 | 1.37±0.39 |
|  | 15 | 10.41±0.61 | 0.89±0.06 | 6.71±2.35 | 1.35±0.44 |
|  | 20 | 10.13±0.84 | 0.84±0.10 | 6.80±2.50 | 1.37±0.47 |
|  | 25 | 10.10±0.81 | 0.84±0.10 | 6.79±2.51 | 1.37±0.47 |

TABLE  S-II

Sensitivity Analysis Results of Key Hyperparameters for the DLE Model on the JPL Dataset

| Parameter | Parameter Setting | Parking Duration SMAPE (%) | Parking Duration MAE (h) | Charging Capacity SMAPE (%) | Charging Capacity MAE (kWh) |
| --- | --- | --- | --- | --- | --- |
| Baseline | Default | 8.79±0.60 | 0.92±0.05 | 8.98±0.33 | 2.88±0.12 |
|  | 0.1 | 8.62±0.54 | 0.90±0.05 | 9.19±0.55 | 2.95±0.07 |
|  | 0.3 | 8.68±0.57 | 0.91±0.05 | 9.21±0.64 | 2.96±0.09 |
|  | 0.5 | 8.72±0.59 | 0.91±0.05 | 9.29±0.70 | 2.98±0.12 |
|  | 0.7 | 8.79±0.60 | 0.92±0.05 | 8.98±0.33 | 2.88±0.12 |
|  | 0.9 | 8.82±0.53 | 0.92±0.05 | 9.26±0.48 | 2.97±0.13 |
|  | 1 | 8.69±0.58 | 0.91±0.05 | 9.18±0.44 | 2.94±0.10 |
|  | 3 | 8.73±0.58 | 0.91±0.05 | 9.24±0.65 | 2.97±0.10 |
|  | 5 | 8.79±0.60 | 0.92±0.05 | 8.98±0.33 | 2.88±0.12 |
|  | 7 | 8.80±0.54 | 0.92±0.05 | 9.22±0.59 | 2.96±0.10 |
|  | 9 | 8.80±0.55 | 0.92±0.05 | 9.25±0.55 | 2.97±0.11 |
|  | 1 | 8.77±0.55 | 0.91±0.05 | 9.22±0.53 | 2.96±0.08 |
|  | 3 | 8.78±0.57 | 0.91±0.06 | 9.28±0.70 | 2.98±0.11 |
|  | 5 | 8.79±0.60 | 0.92±0.05 | 8.98±0.33 | 2.88±0.12 |
|  | 7 | 8.78±0.54 | 0.91±0.05 | 9.26±0.57 | 2.97±0.09 |
|  | 9 | 8.78±0.56 | 0.91±0.05 | 9.30±0.55 | 2.98±0.10 |
|  | 5 | 8.81±0.55 | 0.92±0.05 | 9.18±0.38 | 2.94±0.10 |
|  | 10 | 8.77±0.54 | 0.91±0.06 | 9.15±0.39 | 2.93±0.12 |
|  | 15 | 8.79±0.60 | 0.92±0.05 | 8.98±0.33 | 2.88±0.12 |
|  | 20 | 8.76±0.61 | 0.91±0.05 | 9.28±0.72 | 2.98±0.12 |
|  | 25 | 8.74±0.54 | 0.91±0.05 | 9.28±0.49 | 2.98±0.07 |