# Supplementary Material

## A. Sensitivity Analysis of Key Hyperparameters

To further evaluate the influence and robustness of the key hyperparameter settings in the DLE model, one-factor-at-a-time sensitivity analyses were conducted on the Caltech and JPL datasets. Specifically, the effects of the adjustment-factor control parameter, kurtosis weighting coefficient, skewness weighting coefficientand warm-up roundson model performance were examined. In each experiment, only one parameter was varied while the remaining parameters were kept at their default values. The results are reported as the mean value and maximum deviation across the six training windows.

As shown in Table S-I, on the Caltech dataset, when,, or, the parking-duration MAE decreases from 0.89 h under the default setting to approximately 0.84 h. However, the corresponding MAE and SMAPE for charging energy increase to varying degrees. For example, when, the charging-energy MAE increases from 1.35 kWh to 1.37 kWh. Table S-II shows that the JPL dataset exhibits a similar trend. Some parameter settings slightly reduce the parking-duration error; for instance, when, the parking-duration MAE decreases to 0.90 h. Nevertheless, the charging-energy MAE increases to 2.95 kWh, which is inferior to the 2.88 kWh obtained under the default parameter setting.

Considering the results across the two prediction targets of the two datasets, the default parameter combination does not necessarily yield the minimum value for every individual metric, but it provides a more balanced performance between parking-duration and charging-energy prediction. In particular, for charging-energy prediction, this combination maintains relatively low errors on both the Caltech and JPL datasets, while the parking-duration errors remain close to the locally optimal results. Therefore,,,, andare selected as the default hyperparameter settings to balance prediction accuracy and stability across the two targets. The results indicate that the proposed residual-driven adjustment mechanism is relatively insensitive to moderate variations of these control parameters.

TABLE  S-II

Sensitivity Analysis Results of Key Hyperparameters for the DLE Model on the JPL Dataset

| Parameter | Parameter Setting | Parking Duration SMAPE (%) | Parking Duration MAE (h) | Charging Capacity SMAPE (%) | Charging Capacity MAE (kWh) |
| --- | --- | --- | --- | --- | --- |
| Baseline | Default | 8.79 ± 0.60 | 0.92 ± 0.05 | 8.98 ± 0.33 | 2.88 ± 0.12 |
|  | 0.1 | 8.62 ± 0.54 | 0.90 ± 0.05 | 9.19 ± 0.55 | 2.95 ± 0.07 |
|  | 0.3 | 8.68 ± 0.57 | 0.91 ± 0.05 | 9.21 ± 0.64 | 2.96 ± 0.09 |
|  | 0.5 | 8.72 ± 0.59 | 0.91 ± 0.05 | 9.29 ± 0.70 | 2.98 ± 0.12 |
|  | 0.7 | 8.79 ± 0.60 | 0.92 ± 0.05 | 8.98 ± 0.33 | 2.88 ± 0.12 |
|  | 0.9 | 8.82 ± 0.53 | 0.92 ± 0.05 | 9.26 ± 0.48 | 2.97 ± 0.13 |
|  | 1 | 8.69 ± 0.58 | 0.91 ± 0.05 | 9.18 ± 0.44 | 2.94 ± 0.10 |
|  | 3 | 8.73 ± 0.58 | 0.91 ± 0.05 | 9.24 ± 0.65 | 2.97 ± 0.10 |
|  | 5 | 8.79 ± 0.60 | 0.92 ± 0.05 | 8.98 ± 0.33 | 2.88 ± 0.12 |
|  | 7 | 8.80 ± 0.54 | 0.92 ± 0.05 | 9.22 ± 0.59 | 2.96 ± 0.10 |
|  | 9 | 8.80 ± 0.55 | 0.92 ± 0.05 | 9.25 ± 0.55 | 2.97 ± 0.11 |
|  | 1 | 8.77 ± 0.55 | 0.91 ± 0.05 | 9.22 ± 0.53 | 2.96 ± 0.08 |
|  | 3 | 8.78 ± 0.57 | 0.91 ± 0.06 | 9.28 ± 0.70 | 2.98 ± 0.11 |
|  | 5 | 8.79 ± 0.60 | 0.92 ± 0.05 | 8.98 ± 0.33 | 2.88 ± 0.12 |
|  | 7 | 8.78 ± 0.54 | 0.91 ± 0.05 | 9.26 ± 0.57 | 2.97 ± 0.09 |
|  | 9 | 8.78 ± 0.56 | 0.91 ± 0.05 | 9.30 ± 0.55 | 2.98 ± 0.10 |
|  | 5 | 8.81 ± 0.55 | 0.92 ± 0.05 | 9.18 ± 0.38 | 2.94 ± 0.10 |
|  | 10 | 8.77 ± 0.54 | 0.91 ± 0.06 | 9.15 ± 0.39 | 2.93 ± 0.12 |
|  | 15 | 8.79 ± 0.60 | 0.92 ± 0.05 | 8.98 ± 0.33 | 2.88 ± 0.12 |
|  | 20 | 8.76 ± 0.61 | 0.91 ± 0.05 | 9.28 ± 0.72 | 2.98 ± 0.12 |
|  | 25 | 8.74 ± 0.54 | 0.91 ± 0.05 | 9.28 ± 0.49 | 2.98 ± 0.07 |

TABLE  S-I

Sensitivity Analysis Results of Key Hyperparameters for the DLE Model on the Caltech Dataset

| Parameter | Parameter Setting | Parking Duration SMAPE (%) | Parking Duration MAE (h) | Charging Capacity SMAPE (%) | Charging Capacity MAE (kWh) |
| --- | --- | --- | --- | --- | --- |
| Baseline | Default | 10.41 ± 0.61 | 0.89 ± 0.06 | 6.71 ± 2.35 | 1.35 ± 0.44 |
|  | 0.1 | 10.08 ± 0.82 | 0.84 ± 0.09 | 6.75 ± 2.60 | 1.36 ± 0.49 |
|  | 0.3 | 10.14 ± 0.70 | 0.85 ± 0.08 | 6.82 ± 2.66 | 1.37 ± 0.50 |
|  | 0.5 | 10.20 ± 0.77 | 0.85 ± 0.09 | 6.84 ± 2.50 | 1.38 ± 0.47 |
|  | 0.7 | 10.41 ± 0.61 | 0.89 ± 0.06 | 6.71 ± 2.35 | 1.35 ± 0.44 |
|  | 0.9 | 10.20 ± 0.77 | 0.85 ± 0.09 | 6.83 ± 2.11 | 1.38 ± 0.39 |
|  | 1 | 10.17 ± 0.74 | 0.85 ± 0.09 | 6.80 ± 2.38 | 1.37 ± 0.44 |
|  | 3 | 10.20 ± 0.76 | 0.85 ± 0.09 | 6.83 ± 2.44 | 1.38 ± 0.45 |
|  | 5 | 10.41 ± 0.61 | 0.89 ± 0.06 | 6.71 ± 2.35 | 1.35 ± 0.44 |
|  | 7 | 10.21 ± 0.75 | 0.85 ± 0.09 | 6.76 ± 2.16 | 1.36 ± 0.40 |
|  | 9 | 10.22 ± 0.83 | 0.85 ± 0.10 | 6.82 ± 2.22 | 1.37 ± 0.41 |
|  | 1 | 10.19 ± 0.70 | 0.85 ± 0.08 | 6.85 ± 2.43 | 1.38 ± 0.45 |
|  | 3 | 10.19 ± 0.70 | 0.85 ± 0.08 | 6.84 ± 2.45 | 1.38 ± 0.46 |
|  | 5 | 10.41 ± 0.61 | 0.89 ± 0.06 | 6.71 ± 2.35 | 1.35 ± 0.44 |
|  | 7 | 10.20 ± 0.77 | 0.85 ± 0.09 | 6.75 ± 2.14 | 1.36 ± 0.39 |
|  | 9 | 10.20 ± 0.76 | 0.85 ± 0.09 | 6.81 ± 2.29 | 1.37 ± 0.42 |
|  | 5 | 10.23 ± 0.75 | 0.86 ± 0.09 | 6.75 ± 1.98 | 1.36 ± 0.36 |
|  | 10 | 10.25 ± 0.72 | 0.86 ± 0.09 | 6.80 ± 2.13 | 1.37 ± 0.39 |
|  | 15 | 10.41 ± 0.61 | 0.89 ± 0.06 | 6.71 ± 2.35 | 1.35 ± 0.44 |
|  | 20 | 10.13 ± 0.84 | 0.84 ± 0.10 | 6.80 ± 2.50 | 1.37 ± 0.47 |
|  | 25 | 10.10 ± 0.81 | 0.84 ± 0.10 | 6.79 ± 2.51 | 1.37 ± 0.47 |

B. Temporal Prediction Performance

To further examine the predictive performance of DLE from a temporal perspective, time-series comparisons between the measured and predicted values were conducted for the Caltech and JPL datasets. Fig. S1 present the charging energy and parking duration prediction curves across the six training windows. Overall, the predicted curves follow the main temporal variation of the measured observations and capture the recurring fluctuation patterns across different test periods. The agreement is more evident in the central ranges of the observations, whereas larger deviations mainly occur around abrupt peaks and troughs.

## C. Dynamic Adjustment of Internal Hyperparameters

To visualize the internal training dynamics of DLE, the evolution of tree depth and the regularization coefficient over optimization iterations is further examined. Fig. S2 show the corresponding dynamic curves for charging-energy and parking-duration prediction under the 30-day and 240-day training windows. The tree depth is adjusted in discrete steps and subsequently remains within a stable range, while the regularization coefficient undergoes gradual adjustment before reaching a relatively stable level. These trajectories indicate that the proposed dynamic hyperparameter tuning strategy continuously responds to the residual-driven optimization process while avoiding unbounded growth in model complexity. The different trajectories observed across training windows and prediction targets further demonstrate that the internal hyperparameters are adaptively adjusted according to the corresponding prediction task and data characteristics

Fig. S2.  Dynamic adjustment of tree depth and regularization coefficient during optimization under different training windows: (a) charging energy, 30-day window; (b) parking duration, 30-day window; (c) charging energy, 240-day window; and (d) parking duration, 240-day window.

Fig. S1.  Time-series prediction performance of DLE on the Caltech and JPL datasets: (a) Caltech charging energy, (b) Caltech parking duration, (c) JPL charging energy, and (d) JPL parking duration.

.